CDA 4203/4203L

Spring 2026

# Computer System Design

Final Project Report

Due by 11:59PM, 4 May

| Today’s Date: | 5/4/2026 Team 7 |  |
| --- | --- | --- |
| Team Member Names: Your U Numbers: | Avery Boydston Fredy Perez Jason Maya Matthew Elza | U33299663 U28807695 U60127647 U73922228 |
| Work Distribution | Briefly explain each team member’s contribution. Avery Boydston – Code, Document, Block Diagram Fredy Perez – Code, Board Work, Document Jason Maya – Code, Board Work, Video Matthew Elza – Code, Board Work, Video |  |

Feedback:  Your feedback is extremely important to improve the project for future course offerings.

| Total Number of Person Hours Spent: | Estimate the number of hours spent by each team member and add the four numbers. Cumulatively 25+ |
| --- | --- |
| Exercise Difficulty: (Easy, Average, Hard) | Hard |
| Issues You ran into: | List all problem/issues you faced while doing this project. (please use bulletized list) - Getting the code to work with the board - Time management with finals week and other classes |
| Any Suggestions to improve this project: |  |
| Any Other Feedback: |  |

# Put the link to your demonstration Video here (on YouTube, etc.). Make sure you show as much as you could achieve in the Video.

https://usfedu-my.sharepoint.com/:v:/g/personal/jasonmaya_usf_edu/IQCGDNUYT_2aQZ-cjZDwC6H5AZ2V5JiJfxiZvGnK0gAxbNE?nav=eyJyZWZlcnJhbEluZm8iOnsicmVmZXJyYWxBcHAiOiJPbmVEcml2ZUZvckJ1c2luZXNzIiwicmVmZXJyYWxBcHBQbGF0Zm9ybSI6IldlYiIsInJlZmVycmFsTW9kZSI6InZpZXciLCJyZWZlcnJhbFZpZXciOiJNeUZpbGVzTGlua0NvcHkifX0&e=1cf7M8

# Overview

Describe the overall functionality of the system. Describe any and all relevant interfaces. Maximum 1 page.

The Audio Message Recorder lets a user record, store, play back, pause, resume, stop, and delete spoken audio messages through a serial terminal. The system supports four independent message slots, each backed by a 4 MB region of the onboard RAM. At the codec sample rate this works out to roughly twenty-two seconds of mono audio per slot.

A PicoBlaze KCPSM6 soft microcontroller runs the user-facing menu. On startup the firmware prints a welcome banner and a five-option menu (Play, Record, Delete, Delete All, Volume Control). After the user picks an action, the firmware walks them through prompts to choose a slot, confirm an operation, or set a volume level from 0 to 7. The firmware also drives an LED status register where the upper nibble shows the current volume setting and the lower nibble lights up the slots that currently hold valid recordings.

The recording and playback work is handled by a finite-state machine running in the RAM clock domain at 37.5 MHz. It accepts commands from PicoBlaze through clock-domain-crossing toggle synchronizers, drives the RAM controller, and exchanges sixteen-bit audio samples with the codec. Microphone audio enters the codec, gets registered into the FSM domain on each codec sample-end pulse, and is written byte-by-byte at the slot's base address. During playback the FSM reads two bytes per sample, applies a sign-extending arithmetic shift for volume scaling, so volume 0 attenuates by seven bits and volume 7 is unity gain, and gives the sixteen-bit result back to the codec when sample_req fires.

Three distinct clock domains coordinate the design. The 100 MHz oscillator feeds the RAM controller PLL whose 37.5 MHz output drives the main FSM. A custom PLL wrapper that we wrote (clk_wiz_37to100.v) brings that rate back up to 100 MHz for PicoBlaze and the UART. A second clock wizard derives 50 MHz (codec I2C configuration) and 11.2896 MHz (codec MCLK) from the 100 MHz domain. Toggle synchronizers and double-flop chains handle every signal that crosses between domains.

# System Block Diagram

Show a detailed diagram of the system design. This should include all major blocks and ICs, and show all datapaths and control signals.

# Codes

Include source codes. The length of this will vary widely; depending on how much of your design’s functionality is in software.

1. audio_recorder_top.v

`timescale 1ns / 1ps

// CDA 4203 Final Project -- Audio Message Recorder

// Anvyl Board (Spartan-6 XC6SLX45-3-CSG484)

//

// Clock hierarchy:

//   OSC_100MHz -> ram_interface_wrapper.clk

//   ram_interface_wrapper.clkout (37.5 MHz) -> clk_wiz_37to100 -> pb_clk (100 MHz)

//   pb_clk -> clk_wiz_v3_6 -> main_clk (50 MHz) + audio_clk (11.2896 MHz)

//

// PicoBlaze I/O port map:

//   RD 0x00: UART RX data

//   RD 0x01: uart_data_present [bit 0]

//   RD 0x02: uart_buffer_full  [bit 0]

//   WR 0x03: UART TX data

//   WR 0x04: LED register (8-bit)

//   WR 0x05: FSM command (see CMD_* constants below)

//   RD 0x06: FSM status  (bit0=ram_rdy, 1=playing, 2=recording, 3=paused, 4=done)

//   RD 0x07: valid_slots [bits 3:0]

//   WR 0x08: slot select [bits 1:0]  (write before issuing PLAY/RECORD/DELETE cmd)

//   WR 0x09: volume      [bits 2:0]  (write before issuing SET_VOLUME cmd)

//   RD 0x0A: button states BTN[3:0]

module audio_recorder_top (

input  OSC_100MHz,

input  RESET_BTN,       // E6, active-high when pressed

input  [3:0] BTN,       // pushbuttons V5,U4,V3,P4 (active-high)

input  [3:0] SW,        // slide switches P8,P5,P6,R4

output [7:0] LED,

input  rs232_rx,

output rs232_tx,

// Audio CODEC (SSM2603)

inout  AUD_ADCLRCK,

input  AUD_ADCDAT,

inout  AUD_DACLRCK,

output AUD_DACDAT,

output AUD_XCK,

inout  AUD_BCLK,

output AUD_I2C_SCLK,

inout  AUD_I2C_SDAT,

output AUD_MUTE,

// DDR2 RAM

output        hw_ram_rasn,

output        hw_ram_casn,

output        hw_ram_wen,

output [2:0]  hw_ram_ba,

inout         hw_ram_udqs_p,

inout         hw_ram_udqs_n,

inout         hw_ram_ldqs_p,

inout         hw_ram_ldqs_n,

output        hw_ram_udm,

output        hw_ram_ldm,

output        hw_ram_ck,

output        hw_ram_ckn,

output        hw_ram_cke,

output        hw_ram_odt,

output [12:0] hw_ram_ad,

inout  [15:0] hw_ram_dq,

inout         hw_rzq_pin,

inout         hw_zio_pin

);

// ─────────────────────────────────────────────────────────────

// Reset

// ─────────────────────────────────────────────────────────────

wire reset = RESET_BTN;   // active-high

// ─────────────────────────────────────────────────────────────

// Clocks

// ─────────────────────────────────────────────────────────────

wire ram_clk;      // 37.5 MHz from RAM PLL output

wire pb_clk;       // 100 MHz for PicoBlaze + UART

wire main_clk;     // 50 MHz for I2C codec config

wire audio_clk;    // 11.2896 MHz for audio codec

clk_wiz_37to100 cwiz (

.CLK_IN  (ram_clk),

.CLK_OUT (pb_clk),

.LOCKED  ()

);

clk_wiz_v3_6 audio_pll (

.CLK_IN1 (pb_clk),

.CLK_OUT1(main_clk),

.CLK_OUT2(audio_clk),

.RESET   (reset),

.LOCKED  ()

);

// ─────────────────────────────────────────────────────────────

// DDR2 RAM

// Each slot is 4 MB = 2^22 bytes.

// slot_base(i) = i << 22  (26-bit address)

// ─────────────────────────────────────────────────────────────

localparam SLOT_BYTES = 26'h400000;   // 4 MB per slot

reg  [25:0] ram_addr;

reg  [7:0]  ram_din;

reg         ram_wen;

reg         ram_rreq;

reg         ram_rack;

wire [7:0]  ram_dout;

wire        ram_rdy;

wire        ram_data_pres;

ram_interface_wrapper #(.DATA_BYTE_WIDTH(1)) RAM (

.address      (ram_addr),

.data_in      (ram_din),

.write_enable (ram_wen),

.read_request (ram_rreq),

.read_ack     (ram_rack),

.data_out     (ram_dout),

.reset        (reset),

.clk          (OSC_100MHz),   // raw 100 MHz to RAM PLL input

.sys_clk      (ram_clk),      // 37.5 MHz from clkout (feedback)

.clkout       (ram_clk),

.hw_ram_rasn  (hw_ram_rasn),

.hw_ram_casn  (hw_ram_casn),

.hw_ram_wen   (hw_ram_wen),

.hw_ram_ba    (hw_ram_ba),

.hw_ram_udqs_p(hw_ram_udqs_p),

.hw_ram_udqs_n(hw_ram_udqs_n),

.hw_ram_ldqs_p(hw_ram_ldqs_p),

.hw_ram_ldqs_n(hw_ram_ldqs_n),

.hw_ram_udm   (hw_ram_udm),

.hw_ram_ldm   (hw_ram_ldm),

.hw_ram_ck    (hw_ram_ck),

.hw_ram_ckn   (hw_ram_ckn),

.hw_ram_cke   (hw_ram_cke),

.hw_ram_odt   (hw_ram_odt),

.hw_ram_ad    (hw_ram_ad),

.hw_ram_dq    (hw_ram_dq),

.hw_rzq_pin   (hw_rzq_pin),

.hw_zio_pin   (hw_zio_pin),

.rdy          (ram_rdy),

.rd_data_pres (ram_data_pres),

.max_ram_address(),

.ledRAM       ()

);

// ─────────────────────────────────────────────────────────────

// Audio CODEC

// ─────────────────────────────────────────────────────────────

wire [1:0]  sample_end;

wire [1:0]  sample_req;

wire [15:0] audio_input;

reg  [15:0] audio_output_r;    // driven by FSM (ram_clk domain)

assign AUD_XCK  = audio_clk;

assign AUD_MUTE = 1'b1;        // 1 = mute disabled

i2c_av_config codec_cfg (

.clk     (main_clk),

.reset   (reset),

.i2c_sclk(AUD_I2C_SCLK),

.i2c_sdat(AUD_I2C_SDAT),

.status  ()

);

audio_codec codec (

.clk         (audio_clk),

.reset       (reset),

.sample_end  (sample_end),

.sample_req  (sample_req),

.audio_output(audio_output_r),

.audio_input (audio_input),

.channel_sel (2'b10),

.AUD_ADCLRCK (AUD_ADCLRCK),

.AUD_ADCDAT  (AUD_ADCDAT),

.AUD_DACLRCK (AUD_DACLRCK),

.AUD_DACDAT  (AUD_DACDAT),

.AUD_BCLK    (AUD_BCLK)

);

// ─────────────────────────────────────────────────────────────

// Clock-domain crossing: audio <-> ram_clk (37.5 MHz)

// Toggle synchronisers ensure single-cycle pulses are captured.

// ─────────────────────────────────────────────────────────────

// --- Microphone capture (audio_clk -> ram_clk) ---

reg [15:0] rec_sample;          // stable ~22 µs after each sample_end

reg        rec_toggle_aud;      // toggles on each new sample

always @(posedge audio_clk) begin

if (sample_end[1]) begin

rec_sample     <= audio_input;

rec_toggle_aud <= ~rec_toggle_aud;

end

end

reg [2:0] rec_sync;

always @(posedge ram_clk) rec_sync <= {rec_sync[1:0], rec_toggle_aud};

wire new_rec_sample = rec_sync[1] ^ rec_sync[2];

// --- DAC request (audio_clk -> ram_clk) ---

reg req_toggle_aud;

always @(posedge audio_clk) begin

if (sample_req[1]) req_toggle_aud <= ~req_toggle_aud;

end

reg [2:0] req_sync;

always @(posedge ram_clk) req_sync <= {req_sync[1:0], req_toggle_aud};

wire sample_req_pulse = req_sync[1] ^ req_sync[2];

// ─────────────────────────────────────────────────────────────

// PicoBlaze port registers (pb_clk, 100 MHz)

// ─────────────────────────────────────────────────────────────

wire [7:0]  pb_port_id;

wire [7:0]  pb_out_port;

reg  [7:0]  pb_in_port;

wire        pb_read_strobe;

wire        pb_write_strobe;

// Command path to FSM (CDC: pb_clk -> ram_clk)

reg [7:0]   pb_cmd;

reg [1:0]   pb_slot;

reg [2:0]   pb_volume;

reg         cmd_toggle_pb;

always @(posedge pb_clk or posedge reset) begin

if (reset) begin

pb_cmd <= 8'h00; pb_slot <= 2'd0;

pb_volume <= 3'd5; cmd_toggle_pb <= 1'b0;

end else if (pb_write_strobe) begin

case (pb_port_id)

8'h05: begin pb_cmd <= pb_out_port;

if (pb_out_port != 8'h00)

cmd_toggle_pb <= ~cmd_toggle_pb; end

8'h08: pb_slot   <= pb_out_port[1:0];

8'h09: pb_volume <= pb_out_port[2:0];

default: ;

endcase

end

end

// Sync command data to ram_clk (3 stages for toggle, 2 for data)

reg [2:0] cmd_tog_sync;

reg [7:0] cmd_d1, cmd_d2;

reg [1:0] slot_d1, slot_d2;

reg [2:0] vol_d1, vol_d2;

always @(posedge ram_clk) begin

cmd_tog_sync <= {cmd_tog_sync[1:0], cmd_toggle_pb};

cmd_d1  <= pb_cmd;    cmd_d2  <= cmd_d1;

slot_d1 <= pb_slot;   slot_d2 <= slot_d1;

vol_d1  <= pb_volume; vol_d2  <= vol_d1;

end

wire new_cmd   = cmd_tog_sync[1] ^ cmd_tog_sync[2];

wire [7:0] cmd = cmd_d2;

wire [1:0] sel = slot_d2;

wire [2:0] vol = vol_d2;

// ─────────────────────────────────────────────────────────────

// Message metadata (volatile FPGA registers)

// ─────────────────────────────────────────────────────────────

reg [3:0]  msg_valid;

reg [25:0] msg_len [0:3];

// ─────────────────────────────────────────────────────────────

// Main FSM (ram_clk, 37.5 MHz)

// ─────────────────────────────────────────────────────────────

localparam ST_INIT       = 4'd0;

localparam ST_IDLE       = 4'd1;

localparam ST_REC_WAIT   = 4'd2;

localparam ST_REC_WR_HI  = 4'd3;

localparam ST_REC_WR_LO  = 4'd4;

localparam ST_PLAY_RD_HI = 4'd5;

localparam ST_PLAY_WD_HI = 4'd6;

localparam ST_PLAY_RD_LO = 4'd7;

localparam ST_PLAY_WD_LO = 4'd8;

localparam ST_PLAY_WAIT  = 4'd9;

localparam ST_PAUSED     = 4'd10;

// Command constants

localparam CMD_PLAY   = 8'h01;

localparam CMD_RECORD = 8'h02;

localparam CMD_STOP   = 8'h03;

localparam CMD_PAUSE  = 8'h04;

localparam CMD_RESUME = 8'h05;

localparam CMD_DEL    = 8'h06;

localparam CMD_DELALL = 8'h07;

localparam CMD_SETVOL = 8'h08;

reg [3:0]  state;

reg [25:0] cur_addr;

reg [25:0] play_end;

reg [1:0]  active_slot;

reg [2:0]  volume;

reg [7:0]  hi_byte;

reg [15:0] play_sample;

reg        fsm_playing, fsm_recording, fsm_paused, fsm_done;

// Helper: slot base address (each slot = 2^22 bytes)

function [25:0] slot_base;

input [1:0] s;

slot_base = {2'b00, s, 22'b0};

endfunction

always @(posedge ram_clk or posedge reset) begin

if (reset) begin

state        <= ST_INIT;

cur_addr     <= 26'h0;

play_end     <= 26'h0;

active_slot  <= 2'd0;

volume       <= 3'd5;

hi_byte      <= 8'h0;

play_sample  <= 16'h0;

fsm_playing  <= 1'b0;

fsm_recording<= 1'b0;

fsm_paused   <= 1'b0;

fsm_done     <= 1'b0;

msg_valid    <= 4'b0;

msg_len[0]   <= 26'h0;

msg_len[1]   <= 26'h0;

msg_len[2]   <= 26'h0;

msg_len[3]   <= 26'h0;

audio_output_r <= 16'h0;

ram_wen      <= 1'b0;

ram_rreq     <= 1'b0;

ram_rack     <= 1'b0;

ram_addr     <= 26'h0;

ram_din      <= 8'h0;

end else begin

// Default de-assert

ram_wen  <= 1'b0;

ram_rreq <= 1'b0;

ram_rack <= 1'b0;

fsm_done <= 1'b0;

case (state)

ST_INIT: begin

if (ram_rdy) state <= ST_IDLE;

end

//----------------------------------------------

ST_IDLE: begin

fsm_recording <= 1'b0;

fsm_playing   <= 1'b0;

fsm_paused    <= 1'b0;

audio_output_r <= 16'h0;

if (new_cmd) begin

case (cmd)

CMD_PLAY: begin

if (msg_valid[sel] && msg_len[sel] > 26'h0) begin

active_slot <= sel;

cur_addr    <= slot_base(sel);

play_end    <= slot_base(sel) + msg_len[sel];

fsm_playing <= 1'b1;

state       <= ST_PLAY_RD_HI;

end

end

CMD_RECORD: begin

active_slot  <= sel;

cur_addr     <= slot_base(sel);

msg_valid[sel] <= 1'b0;

msg_len[sel]   <= 26'h0;

fsm_recording  <= 1'b1;

state          <= ST_REC_WAIT;

end

CMD_DEL: begin

msg_valid[sel] <= 1'b0;

msg_len[sel]   <= 26'h0;

end

CMD_DELALL: begin

msg_valid  <= 4'b0;

msg_len[0] <= 26'h0; msg_len[1] <= 26'h0;

msg_len[2] <= 26'h0; msg_len[3] <= 26'h0;

end

CMD_SETVOL: volume <= vol;

default: ;

endcase

end

end

//----------------------------------------------

ST_REC_WAIT: begin

if (new_cmd && cmd == CMD_STOP) begin

msg_valid[active_slot] <= (msg_len[active_slot] > 26'h0) ? 1'b1 : 1'b0;

fsm_recording <= 1'b0;

fsm_done      <= 1'b1;

state <= ST_IDLE;

end else if (msg_len[active_slot] >= SLOT_BYTES - 2) begin

// Slot full

msg_valid[active_slot] <= 1'b1;

fsm_recording <= 1'b0;

fsm_done      <= 1'b1;

state <= ST_IDLE;

end else if (new_rec_sample) begin

state <= ST_REC_WR_HI;

end

end

ST_REC_WR_HI: begin

ram_addr <= cur_addr;

ram_din  <= rec_sample[15:8];

ram_wen  <= 1'b1;

state    <= ST_REC_WR_LO;

end

ST_REC_WR_LO: begin

ram_addr <= cur_addr + 26'h1;

ram_din  <= rec_sample[7:0];

ram_wen  <= 1'b1;

cur_addr <= cur_addr + 26'h2;

msg_len[active_slot] <= msg_len[active_slot] + 26'h2;

state <= ST_REC_WAIT;

end

//----------------------------------------------

ST_PLAY_RD_HI: begin

if (cur_addr >= play_end) begin

audio_output_r <= 16'h0;

fsm_playing    <= 1'b0;

fsm_done       <= 1'b1;

state <= ST_IDLE;

end else begin

ram_addr <= cur_addr;

ram_rreq <= 1'b1;

state    <= ST_PLAY_WD_HI;

end

end

ST_PLAY_WD_HI: begin

if (ram_data_pres) begin

hi_byte  <= ram_dout;

ram_rack <= 1'b1;

state    <= ST_PLAY_RD_LO;

end

end

ST_PLAY_RD_LO: begin

ram_addr <= cur_addr + 26'h1;

ram_rreq <= 1'b1;

state    <= ST_PLAY_WD_LO;

end

ST_PLAY_WD_LO: begin

if (ram_data_pres) begin

play_sample <= {hi_byte, ram_dout};

ram_rack    <= 1'b1;

cur_addr    <= cur_addr + 26'h2;

state       <= ST_PLAY_WAIT;

end

end

ST_PLAY_WAIT: begin

// Output pre-fetched sample with volume scaling

case (volume)

3'd0: audio_output_r <= {{7{play_sample[15]}}, play_sample[15:7]};

3'd1: audio_output_r <= {{6{play_sample[15]}}, play_sample[15:6]};

3'd2: audio_output_r <= {{5{play_sample[15]}}, play_sample[15:5]};

3'd3: audio_output_r <= {{4{play_sample[15]}}, play_sample[15:4]};

3'd4: audio_output_r <= {{3{play_sample[15]}}, play_sample[15:3]};

3'd5: audio_output_r <= {{2{play_sample[15]}}, play_sample[15:2]};

3'd6: audio_output_r <= {play_sample[15], play_sample[15:1]};

3'd7: audio_output_r <= play_sample;

endcase

if (new_cmd) begin

if (cmd == CMD_STOP) begin

audio_output_r <= 16'h0;

fsm_playing <= 1'b0;

state <= ST_IDLE;

end else if (cmd == CMD_PAUSE) begin

audio_output_r <= 16'h0;

fsm_paused  <= 1'b1;

fsm_playing <= 1'b0;

state       <= ST_PAUSED;

end else if (cmd == CMD_SETVOL) begin

volume <= vol;

end

end else if (sample_req_pulse) begin

state <= ST_PLAY_RD_HI;

end

end

ST_PAUSED: begin

audio_output_r <= 16'h0;

if (new_cmd) begin

if (cmd == CMD_RESUME) begin

fsm_playing <= 1'b1;

fsm_paused  <= 1'b0;

state       <= ST_PLAY_RD_HI;

end else if (cmd == CMD_STOP) begin

fsm_paused <= 1'b0;

state      <= ST_IDLE;

end

end

end

default: state <= ST_IDLE;

endcase

end

end

// ─────────────────────────────────────────────────────────────

// Status to PicoBlaze (ram_clk -> pb_clk sync)

// ─────────────────────────────────────────────────────────────

wire [7:0] fsm_status_raw  = {3'b0, fsm_done, fsm_paused, fsm_recording, fsm_playing, ram_rdy};

wire [3:0] valid_slots_raw = msg_valid;

reg [7:0] stat_s1, stat_s2;

reg [3:0] vld_s1, vld_s2;

always @(posedge pb_clk) begin

stat_s1 <= fsm_status_raw; stat_s2 <= stat_s1;

vld_s1  <= valid_slots_raw; vld_s2  <= vld_s1;

end

reg [3:0] btn_s1, btn_s2;

always @(posedge pb_clk) begin

btn_s1 <= BTN; btn_s2 <= btn_s1;

end

// ─────────────────────────────────────────────────────────────

// PicoBlaze + UART (pb_clk, 100 MHz)

// ─────────────────────────────────────────────────────────────

wire       uart_data_present, uart_buffer_full;

wire [7:0] uart_rx_data;

wire       uart_write_en;

reg        uart_read_ack;

reg  [7:0] led_r;

rs232_uart UART (

.tx_data_in      (pb_out_port),

.write_tx_data   (uart_write_en),

.tx_buffer_full  (uart_buffer_full),

.rx_data_out     (uart_rx_data),

.read_rx_data_ack(uart_read_ack),

.rx_data_present (uart_data_present),

.rs232_tx        (rs232_tx),

.rs232_rx        (rs232_rx),

.reset           (reset),

.clk             (pb_clk)

);

wire [11:0] pb_addr;

wire [17:0] pb_instr;

wire        pb_bram_en;

kcpsm6 CPU (

.address       (pb_addr),

.instruction   (pb_instr),

.bram_enable   (pb_bram_en),

.port_id       (pb_port_id),

.write_strobe  (pb_write_strobe),

.k_write_strobe(),

.out_port      (pb_out_port),

.read_strobe   (pb_read_strobe),

.in_port       (pb_in_port),

.interrupt     (1'b0),

.interrupt_ack (),

.reset         (reset),

.sleep         (1'b0),

.clk           (pb_clk)

);

program ROM (

.enable     (pb_bram_en),

.address    (pb_addr),

.instruction(pb_instr),

.clk        (pb_clk)

);

// Output port decoding

assign uart_write_en = pb_write_strobe & (pb_port_id == 8'h03);

always @(posedge pb_clk or posedge reset) begin

if (reset) begin

led_r         <= 8'h00;

uart_read_ack <= 1'b0;

end else begin

uart_read_ack <= pb_read_strobe & (pb_port_id == 8'h00);

if (pb_write_strobe && pb_port_id == 8'h04)

led_r <= pb_out_port;

end

end

// Input port mux

always @(*) begin

case (pb_port_id)

8'h00:  pb_in_port = uart_rx_data;

8'h01:  pb_in_port = {7'b0, uart_data_present};

8'h02:  pb_in_port = {7'b0, uart_buffer_full};

8'h06:  pb_in_port = stat_s2;

8'h07:  pb_in_port = {4'h0, vld_s2};

8'h0A:  pb_in_port = {4'h0, ~btn_s2};   // BTNs active-low on Anvyl

default:pb_in_port = 8'h00;

endcase

end

assign LED = led_r;

endmodule

2. recorder_ui.psm

PicoBlaze (KCPSM6) firmware that drives the serial-terminal user interface.

;================================================================

; CDA 4203 Final Project -- Audio Message Recorder

; PicoBlaze (KCPSM6) firmware

;

; Port assignments:

;   RD 00: UART RX data        WR 03: UART TX data

;   RD 01: uart_data_present   WR 04: LEDs

;   RD 02: uart_buffer_full    WR 05: FSM command

;   RD 06: FSM status          WR 08: slot select

;   RD 07: valid_slots[3:0]    WR 09: volume level

;   RD 0A: button states

;

; Register usage:

;   s0  - scratch / char out

;   s1  - scratch / char in

;   s2  - current slot (0-3)

;   s3  - current volume (0-7)

;   s4  - FSM status

;   s5  - valid_slots

;   sF  - preserved by send_char

;================================================================

CONSTANT port_rx,           00

CONSTANT port_data_pres,    01

CONSTANT port_buf_full,     02

CONSTANT port_tx,           03

CONSTANT port_leds,         04

CONSTANT port_cmd,          05

CONSTANT port_status,       06

CONSTANT port_slots,        07

CONSTANT port_slot_sel,     08

CONSTANT port_volume,       09

CONSTANT port_buttons,      0A

CONSTANT cmd_play,          01

CONSTANT cmd_record,        02

CONSTANT cmd_stop,          03

CONSTANT cmd_pause,         04

CONSTANT cmd_resume,        05

CONSTANT cmd_delete,        06

CONSTANT cmd_delete_all,    07

CONSTANT cmd_set_vol,       08

CONSTANT stat_rdy,          01   ; bit 0

CONSTANT stat_playing,      02   ; bit 1

CONSTANT stat_recording,    04   ; bit 2

CONSTANT stat_paused,       08   ; bit 3

CONSTANT stat_done,         10   ; bit 4

CONSTANT ascii_CR,          0D

CONSTANT ascii_LF,          0A

CONSTANT ascii_SPACE,       20

CONSTANT ascii_0,           30

CONSTANT ascii_1,           31

CONSTANT ascii_2,           32

CONSTANT ascii_3,           33

CONSTANT ascii_4,           34

CONSTANT ascii_5,           35

CONSTANT ascii_6,           36

CONSTANT ascii_7,           37

CONSTANT ascii_8,           38

CONSTANT ascii_9,           39

CONSTANT ascii_A,           41

CONSTANT ascii_B,           42

CONSTANT ascii_C,           43

CONSTANT ascii_D,           44

CONSTANT ascii_E,           45

CONSTANT ascii_F,           46

CONSTANT ascii_G,           47

CONSTANT ascii_H,           48

CONSTANT ascii_I,           49

CONSTANT ascii_J,           4A

CONSTANT ascii_K,           4B

CONSTANT ascii_L,           4C

CONSTANT ascii_M,           4D

CONSTANT ascii_N,           4E

CONSTANT ascii_O,           4F

CONSTANT ascii_P,           50

CONSTANT ascii_Q,           51

CONSTANT ascii_R,           52

CONSTANT ascii_S,           53

CONSTANT ascii_T,           54

CONSTANT ascii_U,           55

CONSTANT ascii_V,           56

CONSTANT ascii_W,           57

CONSTANT ascii_X,           58

CONSTANT ascii_Y,           59

CONSTANT ascii_Z,           5A

CONSTANT ascii_a,           61

CONSTANT ascii_b,           62

CONSTANT ascii_c,           63

CONSTANT ascii_d,           64

CONSTANT ascii_e,           65

CONSTANT ascii_f,           66

CONSTANT ascii_g,           67

CONSTANT ascii_h,           68

CONSTANT ascii_i,           69

CONSTANT ascii_j,           6A

CONSTANT ascii_k,           6B

CONSTANT ascii_l,           6C

CONSTANT ascii_m,           6D

CONSTANT ascii_n,           6E

CONSTANT ascii_o,           6F

CONSTANT ascii_p,           70

CONSTANT ascii_q,           71

CONSTANT ascii_r,           72

CONSTANT ascii_s,           73

CONSTANT ascii_t,           74

CONSTANT ascii_u,           75

CONSTANT ascii_v,           76

CONSTANT ascii_w,           77

CONSTANT ascii_x,           78

CONSTANT ascii_y,           79

CONSTANT ascii_z,           7A

CONSTANT ascii_EXCLAIM,     21

CONSTANT ascii_COLON,       3A

CONSTANT ascii_SEMI,        3B

CONSTANT ascii_MINUS,       2D

CONSTANT ascii_SLASH,       2F

CONSTANT ascii_LPAREN,      28

CONSTANT ascii_RPAREN,      29

CONSTANT ascii_EQUAL,       3D

CONSTANT ascii_GREATER,     3E

CONSTANT ascii_LESS,        3C

CONSTANT ascii_STAR,        2A

CONSTANT ascii_HASH,        23

CONSTANT ascii_UNDER,       5F

CONSTANT ascii_QUESTION,    3F

CONSTANT ascii_COMMA,       2C

CONSTANT ascii_PERIOD,      2E

;================================================================

; Start up

;================================================================

cold_start:

LOAD     s2, ascii_0          ; default slot = 0

LOAD     s3, ascii_5          ; default volume = 5

; Wait for RAM to be ready

wait_ram:   INPUT    s4, port_status

TEST     s4, stat_rdy

JUMP     Z, wait_ram

; Set default volume

LOAD     s0, 05

OUTPUT   s0, port_volume

LOAD     s0, cmd_set_vol

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

; Print welcome banner

CALL     print_crlf

CALL     print_banner

CALL     print_menu

;================================================================

; Main menu loop

;================================================================

main_menu:

; Refresh slot status

INPUT    s5, port_slots

; Update LEDs: upper nibble = volume, lower nibble = valid slots

LOAD     s0, s5

OUTPUT   s0, port_leds

; Print prompt

CALL     print_prompt

; Wait for key

CALL     read_char           ; result in s1

; Handle choices

COMPARE  s1, ascii_1

JUMP     Z, do_play

COMPARE  s1, ascii_2

JUMP     Z, do_record

COMPARE  s1, ascii_3

JUMP     Z, do_delete

COMPARE  s1, ascii_4

JUMP     Z, do_delete_all

COMPARE  s1, ascii_5

JUMP     Z, do_volume

; Echo unknown + newline

LOAD     s0, s1

CALL     send_char

CALL     print_crlf

LOAD     s0, ascii_QUESTION

CALL     send_char

CALL     print_crlf

JUMP     main_menu

;================================================================

; 1) Play a message

;================================================================

do_play:

LOAD     s0, ascii_1

CALL     send_char

CALL     print_crlf

CALL     print_str_play_hdr

INPUT    s5, port_slots

CALL     print_slots

CALL     print_str_slot_prompt

CALL     read_slot           ; result in s2

; Check slot valid

LOAD     s0, 01

LOAD     sF, s2

play_check_valid:

COMPARE  sF, 00

JUMP     Z, play_check_done

SL0      s0

SUB      sF, 01

JUMP     play_check_valid

play_check_done:

TEST     s5, s0

JUMP     Z, play_empty_slot

; Send play command

LOAD     s0, s2

OUTPUT   s0, port_slot_sel

LOAD     s0, cmd_play

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_playing

CALL     play_loop

JUMP     main_menu

play_empty_slot:

CALL     print_str_no_msg

JUMP     main_menu

;================================================================

; Play loop: wait for done / handle pause / stop

;================================================================

play_loop:

play_wait:

INPUT    s4, port_status

; Done playing?

TEST     s4, stat_done

JUMP     NZ, play_done

; Still playing or paused?

TEST     s4, stat_playing

JUMP     NZ, play_check_keys

TEST     s4, stat_paused

JUMP     Z, play_done        ; neither: must have stopped

play_check_keys:

; Check UART for 'p' (pause), 'r' (resume), 's' (stop)

INPUT    s0, port_data_pres

TEST     s0, 01

JUMP     Z, play_wait

CALL     read_char

COMPARE  s1, ascii_p

JUMP     Z, play_do_pause

COMPARE  s1, ascii_r

JUMP     Z, play_do_resume

COMPARE  s1, ascii_s

JUMP     Z, play_do_stop

JUMP     play_wait

play_do_pause:

LOAD     s0, cmd_pause

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_paused

JUMP     play_wait

play_do_resume:

LOAD     s0, cmd_resume

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_playing

JUMP     play_wait

play_do_stop:

LOAD     s0, cmd_stop

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_stopped

play_done:

RETURN

;================================================================

; 2) Record a message

;================================================================

do_record:

LOAD     s0, ascii_2

CALL     send_char

CALL     print_crlf

CALL     print_str_rec_hdr

INPUT    s5, port_slots

CALL     print_slots

CALL     print_str_slot_prompt

CALL     read_slot           ; result in s2

LOAD     s0, s2

OUTPUT   s0, port_slot_sel

LOAD     s0, cmd_record

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_recording

record_loop:

INPUT    s4, port_status

TEST     s4, stat_recording

JUMP     Z, record_done

; Check UART for Enter or 's' (UART-only, BTNs unreliable on Anvyl)

INPUT    s0, port_data_pres

TEST     s0, 01

JUMP     Z, record_loop

CALL     read_char

COMPARE  s1, ascii_CR

JUMP     Z, record_stop

COMPARE  s1, ascii_s

JUMP     Z, record_stop

JUMP     record_loop

record_stop:

LOAD     s0, cmd_stop

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

record_wait_done:

INPUT    s4, port_status

TEST     s4, stat_recording

JUMP     NZ, record_wait_done

record_done:

CALL     print_str_saved

JUMP     main_menu

;================================================================

; 3) Delete a message

;================================================================

do_delete:

LOAD     s0, ascii_3

CALL     send_char

CALL     print_crlf

CALL     print_str_del_hdr

INPUT    s5, port_slots

CALL     print_slots

CALL     print_str_slot_prompt

CALL     read_slot           ; result in s2

CALL     print_str_confirm

CALL     read_char

COMPARE  s1, ascii_y

JUMP     NZ, delete_cancel

LOAD     s0, s2

OUTPUT   s0, port_slot_sel

LOAD     s0, cmd_delete

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_deleted

JUMP     main_menu

delete_cancel:

CALL     print_str_cancel

JUMP     main_menu

;================================================================

; 4) Delete all messages

;================================================================

do_delete_all:

LOAD     s0, ascii_4

CALL     send_char

CALL     print_crlf

CALL     print_str_delall_hdr

CALL     print_str_confirm

CALL     read_char

COMPARE  s1, ascii_y

JUMP     NZ, delall_cancel

LOAD     s0, cmd_delete_all

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_deleted

JUMP     main_menu

delall_cancel:

CALL     print_str_cancel

JUMP     main_menu

;================================================================

; 5) Volume control

;================================================================

do_volume:

LOAD     s0, ascii_5

CALL     send_char

CALL     print_crlf

CALL     print_str_vol_hdr

; Print current volume

LOAD     s0, ascii_C

CALL     send_char

LOAD     s0, ascii_u

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_COLON

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, s3

CALL     send_char

CALL     print_crlf

CALL     print_str_vol_prompt

CALL     read_char

COMPARE  s1, ascii_0

JUMP     C, vol_invalid

COMPARE  s1, ascii_8

JUMP     NC, vol_invalid

LOAD     s3, s1              ; store ASCII '0'..'7'

LOAD     s0, s1

LOAD     s1, ascii_0

SUB      s0, s1              ; convert to 0-7

OUTPUT   s0, port_volume

LOAD     s0, cmd_set_vol

OUTPUT   s0, port_cmd

LOAD     s0, 00

OUTPUT   s0, port_cmd

CALL     print_str_done

JUMP     main_menu

vol_invalid:

CALL     print_str_invalid

JUMP     main_menu

;================================================================

; Slot reader: reads a digit 0-3, stores raw digit in s2

;================================================================

read_slot:

read_slot_wait:

CALL     read_char

COMPARE  s1, ascii_0

JUMP     C, read_slot_wait

COMPARE  s1, ascii_4

JUMP     NC, read_slot_wait

LOAD     s0, s1

CALL     send_char

CALL     print_crlf

LOAD     s2, s1

LOAD     s0, s1

LOAD     s1, ascii_0

SUB      s0, s1              ; convert ASCII to 0-3

LOAD     s2, s0

RETURN

;================================================================

; Print slot availability

;================================================================

print_slots:

LOAD     s0, ascii_S

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_COLON

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

; Print 0,1,2,3 with [used]/[empty]

; NOTE: sF is clobbered by send_char, use s6/s7 for state.

LOAD     s6, 00

LOAD     s7, 01

print_slot_loop:

COMPARE  s6, 04

JUMP     Z, print_slots_done

LOAD     s0, ascii_0

ADD      s0, s6

CALL     send_char

LOAD     s0, ascii_EQUAL

CALL     send_char

TEST     s5, s7

JUMP     Z, print_slot_empty

LOAD     s0, ascii_O

CALL     send_char

LOAD     s0, ascii_K

CALL     send_char

JUMP     print_slot_next

print_slot_empty:

LOAD     s0, ascii_MINUS

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

print_slot_next:

LOAD     s0, ascii_SPACE

CALL     send_char

ADD      s6, 01

SL0      s7

JUMP     print_slot_loop

print_slots_done:

CALL     print_crlf

RETURN

;================================================================

; String literal subroutines

;================================================================

print_crlf:

LOAD     s0, ascii_CR

CALL     send_char

LOAD     s0, ascii_LF

CALL     send_char

RETURN

print_banner:

LOAD     s0, ascii_STAR

CALL     send_char

CALL     send_char

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_A

CALL     send_char

LOAD     s0, ascii_u

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_i

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_R

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_c

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_STAR

CALL     send_char

CALL     send_char

CALL     send_char

CALL     print_crlf

RETURN

print_menu:

LOAD     s0, ascii_1

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_P

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_y

CALL     send_char

CALL     print_crlf

LOAD     s0, ascii_2

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_R

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_c

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

CALL     print_crlf

LOAD     s0, ascii_3

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_D

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

CALL     print_crlf

LOAD     s0, ascii_4

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_D

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_A

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

CALL     print_crlf

LOAD     s0, ascii_5

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_V

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_u

CALL     send_char

LOAD     s0, ascii_m

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

CALL     print_crlf

RETURN

print_prompt:

LOAD     s0, ascii_GREATER

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

RETURN

print_str_slot_prompt:

LOAD     s0, ascii_S

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_LPAREN

CALL     send_char

LOAD     s0, ascii_0

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

LOAD     s0, ascii_3

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_COLON

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

RETURN

print_str_play_hdr:

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_P

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_y

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

CALL     print_crlf

RETURN

print_str_rec_hdr:

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_R

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_c

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

CALL     print_crlf

RETURN

print_str_del_hdr:

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_D

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

CALL     send_char

CALL     print_crlf

RETURN

print_str_delall_hdr:

LOAD     s0, ascii_D

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_A

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_QUESTION

CALL     send_char

CALL     print_crlf

RETURN

print_str_confirm:

LOAD     s0, ascii_C

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_f

CALL     send_char

LOAD     s0, ascii_i

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_m

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_LPAREN

CALL     send_char

LOAD     s0, ascii_y

CALL     send_char

LOAD     s0, ascii_SLASH

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

LOAD     s0, ascii_COLON

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

RETURN

print_str_playing:

LOAD     s0, ascii_P

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_y

CALL     send_char

LOAD     s0, ascii_i

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_g

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_LPAREN

CALL     send_char

LOAD     s0, ascii_p

CALL     send_char

LOAD     s0, ascii_SLASH

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_SLASH

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

CALL     print_crlf

RETURN

print_str_paused:

LOAD     s0, ascii_P

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_u

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_stopped:

LOAD     s0, ascii_S

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_p

CALL     send_char

LOAD     s0, ascii_p

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_recording:

LOAD     s0, ascii_R

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_c

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_i

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_g

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_LPAREN

CALL     send_char

LOAD     s0, ascii_E

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_r

CALL     send_char

LOAD     s0, ascii_SLASH

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_t

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_p

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

CALL     print_crlf

RETURN

print_str_saved:

LOAD     s0, ascii_S

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_v

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_deleted:

LOAD     s0, ascii_D

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_cancel:

LOAD     s0, ascii_C

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_c

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_no_msg:

LOAD     s0, ascii_N

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_m

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_s

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_g

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_vol_hdr:

LOAD     s0, ascii_V

CALL     send_char

LOAD     s0, ascii_o

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_u

CALL     send_char

LOAD     s0, ascii_m

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

LOAD     s0, ascii_LPAREN

CALL     send_char

LOAD     s0, ascii_0

CALL     send_char

LOAD     s0, ascii_MINUS

CALL     send_char

LOAD     s0, ascii_7

CALL     send_char

LOAD     s0, ascii_RPAREN

CALL     send_char

CALL     print_crlf

RETURN

print_str_vol_prompt:

LOAD     s0, ascii_N

CALL     send_char

LOAD     s0, ascii_e

CALL     send_char

LOAD     s0, ascii_w

CALL     send_char

LOAD     s0, ascii_COLON

CALL     send_char

LOAD     s0, ascii_SPACE

CALL     send_char

RETURN

print_str_done:

LOAD     s0, ascii_O

CALL     send_char

LOAD     s0, ascii_K

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

print_str_invalid:

LOAD     s0, ascii_I

CALL     send_char

LOAD     s0, ascii_n

CALL     send_char

LOAD     s0, ascii_v

CALL     send_char

LOAD     s0, ascii_a

CALL     send_char

LOAD     s0, ascii_l

CALL     send_char

LOAD     s0, ascii_i

CALL     send_char

LOAD     s0, ascii_d

CALL     send_char

LOAD     s0, ascii_PERIOD

CALL     send_char

CALL     print_crlf

RETURN

;================================================================

; Subroutines: UART I/O

;================================================================

; read_char: block until a character is received, return in s1

read_char:

INPUT    s0, port_data_pres

TEST     s0, 01

JUMP     Z, read_char

INPUT    s1, port_rx

RETURN

; send_char: send byte in s0, wait if TX buffer full

send_char:

INPUT    sF, port_buf_full

TEST     sF, 01

JUMP     NZ, send_char

OUTPUT   s0, port_tx

RETURN

3. audio_recorder.ucf

constraints file with all pin assignments for the Anvyl board (oscillator, buttons, LEDs, switches, UART, audio codec, and DDR2 RAM), required DDR2 timing constraints, and a CLOCK_DEDICATED_ROUTE override that allows our PLL to feed the second clock wizard.

# Anvyl Board -- Audio Message Recorder pin constraints

# System clock (100 MHz crystal)

NET "OSC_100MHz" LOC=D11;

NET "OSC_100MHz" TNM_NET = sys_clk_pin;

TIMESPEC TS_sys_clk_pin = PERIOD sys_clk_pin 100000 kHz;

# Reset button

NET "RESET_BTN" LOC=E6;

# Push buttons (bank 3 = 2.5V VCCIO on Anvyl) - PULLUP because they're active-low

NET "BTN[0]" LOC=V5 | IOSTANDARD=LVCMOS25 | PULLUP;

NET "BTN[1]" LOC=U4 | IOSTANDARD=LVCMOS25 | PULLUP;

NET "BTN[2]" LOC=V3 | IOSTANDARD=LVCMOS25 | PULLUP;

NET "BTN[3]" LOC=P4 | IOSTANDARD=LVCMOS25 | PULLUP;

# Slide switches (bank 3, 2.5V)

NET "SW[0]" LOC=P8 | IOSTANDARD=LVCMOS25;

NET "SW[1]" LOC=P5 | IOSTANDARD=LVCMOS25;

NET "SW[2]" LOC=P6 | IOSTANDARD=LVCMOS25;

NET "SW[3]" LOC=R4 | IOSTANDARD=LVCMOS25;

# LEDs - all on bank 3 (1.8V VCCIO on Anvyl)

NET "LED[0]" LOC=W3  | IOSTANDARD=LVCMOS25;

NET "LED[1]" LOC=Y4  | IOSTANDARD=LVCMOS25;

NET "LED[2]" LOC=Y1  | IOSTANDARD=LVCMOS25;

NET "LED[3]" LOC=Y3  | IOSTANDARD=LVCMOS25;

NET "LED[4]" LOC=AB4 | IOSTANDARD=LVCMOS25;

NET "LED[5]" LOC=W1  | IOSTANDARD=LVCMOS25;

NET "LED[6]" LOC=AB3 | IOSTANDARD=LVCMOS25;

NET "LED[7]" LOC=AA4 | IOSTANDARD=LVCMOS25;

# UART

NET "rs232_tx" LOC=T20;

NET "rs232_rx" LOC=T19;

# Audio CODEC SSM2603

NET "AUD_DACDAT"  LOC=A5  | IOSTANDARD=LVCMOS33;

NET "AUD_DACLRCK" LOC=D6  | IOSTANDARD=LVCMOS33;

NET "AUD_ADCDAT"  LOC=C6  | IOSTANDARD=LVCMOS33;

NET "AUD_ADCLRCK" LOC=B6  | IOSTANDARD=LVCMOS33;

NET "AUD_MUTE"    LOC=A15 | IOSTANDARD=LVCMOS33;

NET "AUD_XCK"     LOC=A6  | IOSTANDARD=LVCMOS33;

NET "AUD_BCLK"    LOC=B12 | IOSTANDARD=LVCMOS33;

NET "AUD_I2C_SDAT" LOC=C5 | PULLUP | IOSTANDARD=LVCMOS33;

NET "AUD_I2C_SCLK" LOC=A4 | PULLUP | IOSTANDARD=LVCMOS33;

# DDR2 RAM timing config

CONFIG MCB_PERFORMANCE = STANDARD;

# DDR2 differential clock

NET "hw_ram_ck"  LOC=F2 | IOSTANDARD=DIFF_SSTL2_II;

NET "hw_ram_ckn" LOC=F1 | IOSTANDARD=DIFF_SSTL2_II;

# DDR2 control signals

NET "hw_ram_cke"  LOC=J6;

NET "hw_ram_casn" LOC=P3;

NET "hw_ram_ldm"  LOC=H1;

NET "hw_ram_odt"  LOC=M3;

NET "hw_ram_rasn" LOC=N4;

NET "hw_ram_udm"  LOC=H2;

NET "hw_ram_wen"  LOC=D2;

# DDR2 differential strobe signals

NET "hw_ram_ldqs_n" LOC=L1 | IOSTANDARD=DIFF_SSTL2_II;

NET "hw_ram_ldqs_p" LOC=L3 | IOSTANDARD=DIFF_SSTL2_II;

NET "hw_ram_udqs_n" LOC=T1 | IOSTANDARD=DIFF_SSTL2_II;

NET "hw_ram_udqs_p" LOC=T2 | IOSTANDARD=DIFF_SSTL2_II;

# DDR2 data bus

NET "hw_ram_dq[0]"  LOC=N3;

NET "hw_ram_dq[1]"  LOC=N1;

NET "hw_ram_dq[2]"  LOC=M2;

NET "hw_ram_dq[3]"  LOC=M1;

NET "hw_ram_dq[4]"  LOC=J3;

NET "hw_ram_dq[5]"  LOC=J1;

NET "hw_ram_dq[6]"  LOC=K2;

NET "hw_ram_dq[7]"  LOC=K1;

NET "hw_ram_dq[8]"  LOC=P2;

NET "hw_ram_dq[9]"  LOC=P1;

NET "hw_ram_dq[10]" LOC=R3;

NET "hw_ram_dq[11]" LOC=R1;

NET "hw_ram_dq[12]" LOC=U3;

NET "hw_ram_dq[13]" LOC=U1;

NET "hw_ram_dq[14]" LOC=V2;

NET "hw_ram_dq[15]" LOC=V1;

NET "hw_ram_dq[*]" IOSTANDARD=SSTL2_II;

NET "hw_ram_dq[*]" IN_TERM=NONE;

# DDR2 address bus

NET "hw_ram_ad[0]"  LOC=M5;

NET "hw_ram_ad[1]"  LOC=L4;

NET "hw_ram_ad[2]"  LOC=K3;

NET "hw_ram_ad[3]"  LOC=M4;

NET "hw_ram_ad[4]"  LOC=K5;

NET "hw_ram_ad[5]"  LOC=G3;

NET "hw_ram_ad[6]"  LOC=G1;

NET "hw_ram_ad[7]"  LOC=K4;

NET "hw_ram_ad[8]"  LOC=C3;

NET "hw_ram_ad[9]"  LOC=C1;

NET "hw_ram_ad[10]" LOC=K6;

NET "hw_ram_ad[11]" LOC=B1;

NET "hw_ram_ad[12]" LOC=J4;

NET "hw_ram_ad[*]" IOSTANDARD=SSTL2_II;

# DDR2 bank address

NET "hw_ram_ba[0]" LOC=E3;

NET "hw_ram_ba[1]" LOC=E1;

NET "hw_ram_ba[2]" LOC=D1;

NET "hw_ram_ba[*]" IOSTANDARD=SSTL2_II;

# Required DDR2 timing constraints (from reference project)

NET "*/memc3_wrapper_inst/mcb_ui_top_inst/mcb_raw_wrapper_inst/selfrefresh_mcb_mode" TIG;

NET "*/c?_pll_lock" TIG;

INST "*/memc?_wrapper_inst/mcb_ui_top_inst/mcb_raw_wrapper_inst/gen_term_calib.mcb_soft_calibration_top_inst/mcb_soft_calibration_inst/DONE_SOFTANDHARD_CAL*" TIG;

NET "*/memc?_wrapper_inst/mcb_ui_top_inst/mcb_raw_wrapper_inst/gen_term_calib.mcb_soft_calibration_top_inst/mcb_soft_calibration_inst/CKE_Train" TIG;

# Allow PLL output to drive non-clock load (the audio PLL CLKIN)

PIN "cwiz/clkout_buf.O" CLOCK_DEDICATED_ROUTE = FALSE;

4. clk_wiz_37to100.v

Custom Spartan-6 PLL_BASE wrapper that takes the 37.5 MHz clkout from the RAM interface and produces 100 MHz for PicoBlaze and the UART.

// Clock Wizard: 37.5 MHz input -> 100 MHz output

// Spartan-6 PLL_BASE

// DIVCLK_DIVIDE=1 (Fpfd=37.5MHz, must be >=19MHz for Spartan-6)

// VCO = 37.5 * 16 / 1 = 600 MHz (in valid 400-1080 range)

// Output = VCO / 6 = 100 MHz

`timescale 1ns / 1ps

module clk_wiz_37to100 (

input  CLK_IN,    // 37.5 MHz from RAM clkout

output CLK_OUT,   // 100 MHz

output LOCKED

);

wire CLKFB;

wire CLKOUT0_raw;

PLL_BASE #(

.BANDWIDTH         ("OPTIMIZED"),

.CLK_FEEDBACK      ("CLKFBOUT"),

.COMPENSATION      ("INTERNAL"),

.DIVCLK_DIVIDE     (1),

.CLKFBOUT_MULT     (16),

.CLKFBOUT_PHASE    (0.000),

.CLKOUT0_DIVIDE    (6),

.CLKOUT0_DUTY_CYCLE(0.500),

.CLKOUT0_PHASE     (0.000),

.CLKIN_PERIOD      (26.667),

.REF_JITTER        (0.010),

.RESET_ON_LOSS_OF_LOCK("FALSE")

) pll_inst (

.CLKFBIN  (CLKFB),

.CLKIN    (CLK_IN),

.RST      (1'b0),

.CLKFBOUT (CLKFB),

.CLKOUT0  (CLKOUT0_raw),

.LOCKED   (LOCKED),

.CLKOUT1  (),

.CLKOUT2  (),

.CLKOUT3  (),

.CLKOUT4  (),

.CLKOUT5  ()

);

BUFG clkout_buf (

.I (CLKOUT0_raw),

.O (CLK_OUT)

);

endmodule

5. i2c_av_config.v

Codec configuration LUT from the audio codec starter project. Five register values were changed to make the recorded and played-back audio audible at line-out levels. ADC gains were boosted to +12 dB on both channels, DAC volumes to +6 dB, and the MICBOOST bit was set to enable the +20 dB microphone preamplifier.

module i2c_av_config (

input clk,

input reset,

output i2c_sclk,

inout  i2c_sdat,

output [3:0] status

);

reg [23:0] i2c_data;

reg [15:0] lut_data;

reg [3:0]  lut_index = 4'd0;

parameter LAST_INDEX = 4'ha;

reg  i2c_start = 1'b0;

wire i2c_done;

wire i2c_ack;

i2c_controller control (

.clk (clk),

.i2c_sclk (i2c_sclk),

.i2c_sdat (i2c_sdat),

.i2c_data (i2c_data),

.start (i2c_start),

.done (i2c_done),

.ack (i2c_ack)

);

always @(*) begin

case (lut_index)

4'h0: lut_data <= 16'h0c10; // power on everything except out

4'h1: lut_data <= 16'h001F; // left input  ADC gain = +12dB (was +0dB)

4'h2: lut_data <= 16'h021F; // right input ADC gain = +12dB

4'h3: lut_data <= 16'h047F; // left output DAC vol = +6dB (was +0dB)

4'h4: lut_data <= 16'h067F; // right output DAC vol = +6dB

4'h5: lut_data <= 16'h08d5; // analog path: MICBOOST=1 (+20dB mic preamp)

4'h6: lut_data <= 16'h0a04; // digital path

4'h7: lut_data <= 16'h0e01; // digital IF

4'h8: lut_data <= 16'h1020; // sampling rate

4'h9: lut_data <= 16'h0c00; // power on everything

4'ha: lut_data <= 16'h1201; // activate

default: lut_data <= 16'h0000;

endcase

end

reg [1:0] control_state = 2'b00;

assign status = lut_index;

always @(posedge clk) begin

if (reset) begin

lut_index <= 4'd0;

i2c_start <= 1'b0;

control_state <= 2'b00;

end else begin

case (control_state)

2'b00: begin

i2c_start <= 1'b1;

i2c_data <= {8'h34, lut_data};

control_state <= 2'b01;

end

2'b01: begin

i2c_start <= 1'b0;

control_state <= 2'b10;

end

2'b10: if (i2c_done) begin

if (i2c_ack) begin

if (lut_index == LAST_INDEX)

control_state <= 2'b11;

else begin

lut_index <= lut_index + 1'b1;

control_state <= 2'b00;

end

end else

control_state <= 2'b00;

end

endcase

end

end

endmodule

# Discussion and Conclusions

In this section, discuss how you arrived at your major decisions. Discuss any aspects of your design that you would do differently if you had it all to do over again, and why. Include any and all thoughts you have on the project, how it could have been done differently or better, etc.

In relation to major design decisions, splitting the design across three clock domains was the most consequential architectural decision. The RAM controller is hard-wired to produce a 37.5 MHz user clock, but PicoBlaze and the UART are most easily timed at 100 MHz, and the codec needs an 11.2896 MHz MCLK. Rather than trying to run everything off a single clock, we used the natural rate of each block and crossed signals carefully. PicoBlaze writes commands into a register file in its own 100 MHz domain and toggles a single bit to signal a new command. The 37.5 MHz FSM samples that toggle through a three-stage synchronizer and treats every change as a new command pulse, then reads the data registers (which are guaranteed stable by then) without further hand-shaking. The same toggle-then-data pattern handles audio samples in both directions.

Volume control is implemented with arithmetic shifts of the sixteen-bit signed sample rather than a hardware multiplier. Each volume level shifts by one fewer bit, so volume 0 attenuates by seven bits, volume 5 by two bits (the boot default), and volume 7 outputs the sample unscaled. This costs essentially zero logic compared with a multiplier and preserves the sign of the sample correctly because we replicate the MSB during the shift. Each slot is sized at exactly 4 MB so that the slot base address is just two bits prepended to a twenty-two-bit offset. With sixteen-bit samples at the codec's native rate, a full slot holds about twenty-two seconds of audio. The metadata (which slots are valid and how many bytes each one holds) lives in FPGA registers rather than in RAM, since none of it persists across power cycles anyway.

In more complex future designs some changes could be made differently. Storing message metadata in RAM alongside the audio data, using a fixed header at the top of each slot, would let us survive power cycles and load existing recordings on boot. The current design loses every recording when the board resets. Replacing the polling-based PicoBlaze loop with an interrupt-driven model would let the firmware sleep between user inputs instead of constantly reading the FSM status port. The savings in power and bus activity would be small but the code would be cleaner. We also considered packing two samples per write to the RAM controller (the controller has a 32-bit data path internally) which would halve the bandwidth requirement and free up address space for longer slots, but we judged the added complexity not worth it for this prototype.

Overall the project successfully meets every functional requirement listed in the specification. The system displays a welcome banner and menu on startup, allows the user to record, play, pause, resume, delete, and adjust volume, and stores up to four messages in onboard memory. The most valuable lessons from the project were the practical realities of clock domain crossing, the importance of reading the codec register map carefully so that recorded and played audio reaches usable line levels, and the value of a well-structured top module as the integration point for many heterogeneous IP blocks.